SAFETY DATA SHEET

024365 Verto Concentrate Surface Disinfectant

# SECTION 1: Identification of the substance/mixture and of the company/undertaking

## 1.1. Product identifier

| Product name UFI | : 024365 Verto Concentrate Surface Disinfectant : 76C5-60Y1-U009-QDUU |
| --- | --- |

## 1.2. Relevant identified uses of the substance or mixture and uses advised against

1.2.1. Relevant identified uses Professional use.

1.2.2. Uses advised against

No additional information available

## 1.3. Details of the supplier of product safety information sheet

Supplier 	PARC SUPPLIES LTD

VISTA HOUSE

WHITEHALL POINT

WHITEHALL ROAD

LEEDS

LS12 4RZ

UNITED KINGDOM

Tel: 0113 220 4080        Email: info@parcglobal.com

## 1.4. Emergency telephone number

Emergency telephone 	 	+44 (0)113 220 4080

National emergency telephone    For the emergency services - the ambulance, police and fire services - Tel: 999 /

number 	When you need medical advice or treatment but it is not an emergency - Tel: 111

# SECTION 2: Hazards identification

## 2.1. Classification of the substance or mixture

Classification according to Regulation (EC) No. 1272/2008 [CLP]

Skin corrosion/irritation, Category 2 	H315

# Verto Concentrate Surface Disinfectant

Serious eye damage/eye irritation, Category 1 	H318

Hazardous to the aquatic environment — Chronic Hazard, Category 3 	H412 Full text of H- and EUH-statements: see section 16

Adverse physicochemical, human health and environmental effects Causes severe skin burns and eye damage.

## 2.2. Label elements

Labelling according to Regulation (EC) No. 1272/2008 [CLP]

Hazard pictograms (CLP) 	:

|  | GHS05 |
| --- | --- |
| Signal word (CLP) | : Danger |
| Contains | : BENZALKONIUM CHLORIDE, C9-11 PARETH-8 |
| Hazard statements (CLP) | : H315 - Causes skin irritation. H318 - Causes serious eye damage. H412 - Harmful to aquatic life with long lasting effects. |
| Precautionary statements (CLP) | : P264 - Wash hands, forearms and face thoroughly after handling. P280 - Wear eye protection, protective gloves. P305+P351+P338+P310 - IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. Immediately call a POISON CENTER or doctor. |

The mixture does not contain substance(s) included in the list established in accordance with Article 59(1) of REACH for having endocrine disrupting properties, or is not identified as having endocrine disrupting properties in accordance with the criteria set out in Commission Delegated Regulation (EU) 2017/2100 or Commission Regulation (EU) 2018/605 at a concentration equal to or greater than 0,1 %

## SECTION 3: Composition/information on ingredients

3.1. Substances

Not applicable

### 3.2. Mixtures

| Name | Product identifier | % | Classification according to Regulation (EC) No. 1272/2008 [CLP] |
| --- | --- | --- | --- |
| BENZALKONIUM CHLORIDE | CAS-No.: 68424-85-1 EC-No.: 270-325-2 | 1 – 5 | Acute Tox. 4 (Oral), H302 Acute Tox. 4 (Dermal), H312 Skin Corr. 1, H314 Eye Dam. 1, H318 Aquatic Acute 1, H400 (M=10) Aquatic Chronic 1, H410 |
| C9-11 PARETH-8 | CAS-No.: 68439-46-3 | 1 – 5 | Acute Tox. 4 (Oral), H302 Eye Dam. 1, H318 |

Full text of H- and EUH-statements: see section 16

## SECTION 4: First aid measures

### 4.1. Description of first aid measures

First-aid measures after inhalation 	: Remove person to fresh air and keep comfortable for breathing.

First-aid measures after skin contact : Wash skin with plenty of water. Take off contaminated clothing. If skin irritation occurs: Get medical advice/attention.

First-aid measures after eye contact 	: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. Call a physician immediately.

First-aid measures after ingestion 	: Call a poison center or a doctor if you feel unwell.

### 4.2. Most important symptoms and effects, both acute and delayed

Symptoms/effects after skin contact 	: Irritation.

Symptoms/effects after eye contact 	: Serious damage to eyes.

4.3. Indication of any immediate medical attention and special treatment needed Treat symptomatically.

## SECTION 5: Firefighting measures

5.1. Extinguishing media

Suitable extinguishing media 	: Water spray. Dry powder. Foam. Carbon dioxide.

### 5.2. Special hazards arising from the substance or mixture

Hazardous decomposition products in case of : Toxic fumes may be released. fire

### 5.3. Advice for firefighters

Protection during firefighting 	: Do not attempt to take action without suitable protective equipment. Selfcontained breathing apparatus. Complete protective clothing.

## SECTION 6: Accidental release measures

### 6.1. Personal precautions, protective equipment and emergency procedures

6.1.1. For non-emergency personnel

| Emergency procedures 6.1.2. For emergency responders | : Ventilate spillage area. Avoid contact with skin and eyes. |
| --- | --- |
| Protective equipment | : Do not attempt to take action without suitable protective equipment. For further information refer to section 8: "Exposure controls/personal protection". |

6.2. Environmental precautions

Avoid release to the environment.

### 6.3. Methods and material for containment and cleaning up

Methods for cleaning up 	: Take up liquid spill into absorbent material.

Other information 	: Dispose of materials or solid residues at an authorized site.

6.4. Reference to other sections

For further information refer to section 13.

## SECTION 7: Handling and storage

### 7.1. Precautions for safe handling

Precautions for safe handling 	: Ensure good ventilation of the work station. Avoid contact with skin and eyes. Do not breathe dust/fume/gas/mist/vapours/spray. Wear personal protective equipment.

Hygiene measures 	: Wash contaminated clothing before reuse. Do not eat, drink or smoke when using

this product. Always wash hands after handling the product.

7.2. Conditions for safe storage, including any incompatibilities

Storage conditions 	: Store in a well-ventilated place. Keep cool.

7.3. Specific end use(s)

No additional information available

## SECTION 8: Exposure controls/personal protection

### 8.1. Control parameters

8.1.1 National occupational exposure and biological limit values

No additional information available

8.1.2. Recommended monitoring procedures No additional information available

8.1.3. Air contaminants formed

No additional information available

8.1.4. DNEL and PNEC

No additional information available

8.1.5. Control banding

No additional information available

### 8.2. Exposure controls

8.2.1. Appropriate engineering controls

Appropriate engineering controls:

Ensure good ventilation of the work station. Avoid direct skin contact with product. Identify potential areas for indirect skin contact. Wear gloves (tested to EN374) if hand contact with substance likely. Clean up contamination/spills as soon as they occur. Wash off any skin contamination immediately. Provide basic employee training to prevent / minimise exposures and to report any skin problems that may develop.

8.2.2. Personal protection equipment

Personal protective equipment symbol(s):

8.2.2.1. Eye and face protection

Eye protection:

Safety glasses. EN 166

8.2.2.2. Skin protection

Skin and body protection:

Wear suitable protective clothing

Hand protection:

Disposable gloves. Wear suitable gloves tested to EN374

8.2.2.3. Respiratory protection

Respiratory protection:

In case of insufficient ventilation, wear suitable respiratory equipment

8.2.2.4. Thermal hazards

No additional information available

8.2.3. Environmental exposure controls

Environmental exposure controls: Avoid release to the environment.

## SECTION 9: Physical and chemical properties

### 9.1. Information on basic physical and chemical properties

Physical state 	: Liquid Colour 	: Clear, violet liquid.

Odour 	: Characteristic

Odour threshold 	: No data available

pH 	: 9.2 – 10.1

Relative evaporation rate (butylacetate=1) 	: No data available

Melting point 	: Not applicable

Freezing point 	: No data available

Boiling point 	: No data available

Flash point 	: No data available

Auto-ignition temperature 	: No data available

Decomposition temperature 	: No data available

Flammability (solid, gas) 	: Not applicable

Vapour pressure 	: No data available

Relative vapour density at 20 °C 	: No data available

Relative density 	: 1.004 – 1.009

Solubility 	: No data available

Partition coefficient n-octanol/water (Log Pow) : No data available

| Viscosity, kinematic | : No data available |
| --- | --- |
| Viscosity, dynamic | : No data available |
| Explosive properties | : No data available |
| Oxidising properties | : No data available |
| Explosive limits | : No data available |

9.2. Other information

No additional information available

## SECTION 10: Stability and reactivity

10.1. Reactivity

The product is non-reactive under normal conditions of use, storage and transport.

10.2. Chemical stability

Stable under normal conditions.

10.3. Possibility of hazardous reactions

No dangerous reactions known under normal conditions of use.

10.4. Conditions to avoid

None under recommended storage and handling conditions (see section 7).

10.5. Incompatible materials

Strong acids.

10.6. Hazardous decomposition products

Under normal conditions of storage and use, hazardous decomposition products should not be produced.

## SECTION 11: Toxicological information

### 11.1. Information on hazard classes as defined in Regulation (EC) No 1272/2008

Acute toxicity (oral) 	: Not classified

Acute toxicity (dermal) 	: Not classified

Acute toxicity (inhalation) 	: Not classified

| BENZALKONIUM CHLORIDE (68424-85-1) |  |  |  |
| --- | --- | --- | --- |
| LD50 oral |  | LD50 oral |  |
| LD50 oral |  | LD50 oral |  |
| Skin corrosion/irritation |  | : Not classified pH: 9.2 – 10.1 |  |
| Serious eye damage/irritation |  | : Not classified pH: 9.2 – 10.1 |  |
| Respiratory or skin sensitisation |  | : Not classified |  |
| Germ cell mutagenicity |  | : Not classified |  |
| Carcinogenicity |  | : Not classified |  |
| Reproductive toxicity |  | : Not classified |  |
| STOT-single exposure |  | : Not classified |  |
| STOT-repeated exposure |  | : Not classified |  |
| Aspiration hazard |  | : Not classified |  |

11.2. Information on other hazards

No additional information available

## SECTION 12: Ecological information

### 12.1. Toxicity

Ecology - general 	: Harmful to aquatic life with long lasting effects.

Hazardous to the aquatic environment, short-	: Not classified term (acute)

Hazardous to the aquatic environment, long-	: Harmful to aquatic life with long lasting effects.

term (chronic)

| BENZALKONIUM CHLORIDE (68424-85-1) |  |
| --- | --- |
| LC50 [1] | 1.7 mg/l |
| EC50 [1] | 0.03 mg/l |

### 12.2. Persistence and degradability

| BENZALKONIUM CHLORIDE (68424-85-1) |  |
| --- | --- |
| Chemical oxygen demand (COD) | 1130 g O₂/g substance |
| Biodegradation | 60 % |

12.3. Bioaccumulative potential

No additional information available

12.4. Mobility in soil

No additional information available

12.5. Results of PBT and vPvB assessment

No additional information available

12.6. Endocrine disrupting properties

No additional information available

12.7. Other adverse effects

No additional information available

## SECTION 13: Disposal considerations

### 13.1. Waste treatment methods

Waste treatment methods 	: Dispose of contents/container in accordance with licensed collector’s sorting

instructions.

## SECTION 14: Transport information

In accordance with ADR / IMDG / IATA / ADN / RID

| ADR | IMDG | IATA | ADN | RID |
| --- | --- | --- | --- | --- |
| 14.1. UN number or ID number |  |  |  |  |
| Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| 14.2. UN proper shipping name |  |  |  |  |
| Not applicable Not applicable |  | Not applicable Not applicable Not applicable |  |  |
| 14.3. Transport hazard class(es) |  |  |  |  |
| Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| 14.4. Packing group |  |  |  |  |
| Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| 14.5. Environmental hazards |  |  |  |  |
| Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| No supplementary information available |  |  |  |  |

14.6. Special precautions for user

Overland transport Not applicable

Transport by sea Not applicable

Air transport

Not applicable

Inland waterway transport

Not applicable

Rail transport

Not applicable

14.7. Maritime transport in bulk according to IMO instruments

Not applicable

## SECTION 15: Regulatory information

### 15.1. Safety, health and environmental regulations/legislation specific for the substance or mixture

15.1.1. EU-Regulations

REACH Annex XVII (Restriction List)

Contains no REACH substances with Annex XVII restrictions

REACH Annex XIV (Authorisation List)

Contains no REACH Annex XIV substances

REACH Candidate List (SVHC)

Contains no substance on the REACH candidate list

PIC Regulation (Prior Informed Consent)

Contains no substance subject to Regulation (EU) No 649/2012 of the European Parliament and of the Council of 4 July 2012 concerning the export and import of hazardous chemicals. POP Regulation (Persistent Organic Pollutants)

Contains no substance subject to Regulation (EU) No 2019/1021 of the European Parliament and of the Council of 20 June 2019 on persistent organic pollutants

Ozone Regulation (1005/2009)

Contains no substance subject to REGULATION (EU) No 1005/2009 OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL of 16 September 2009 on substances that deplete the ozone layer.

Explosives Precursors Regulation (2019/1148)

Contains no substance subject to Regulation (EU) 2019/1148 of the European Parliament and of the Council of 20 June 2019 on the marketing and use of explosives precursors. Drug Precursors Regulation (273/2004)

Contains no substance subject to Regulation (EC) 273/2004 of the European Parliament and of the Council of 11 February 2004 on the manufacture and the placing on market of certain substances used in the illicit manufacture of narcotic drugs and psychotropic substances.

15.1.2. National regulations

No additional information available

15.2. Chemical safety assessment

No chemical safety assessment has been carried out

## SECTION 16: Other information

| Abbreviations and acronyms: |  |
| --- | --- |
| ADN | European Agreement concerning the International Carriage of Dangerous Goods by Inland Waterways |
| ADR | European Agreement concerning the International Carriage of Dangerous Goods by Road |
| ATE | Acute Toxicity Estimate |
| BCF | Bioconcentration factor |
| BLV | Biological limit value |
| BOD | Biochemical oxygen demand (BOD) |
| COD | Chemical oxygen demand (COD) |
| DMEL | Derived Minimal Effect level |
| DNEL | Derived-No Effect Level |

| EC-No. | European Community number |
| --- | --- |
| EC50 | Median effective concentration |
| EN | European Standard |
| IARC | International Agency for Research on Cancer |
| IATA | International Air Transport Association |
| IMDG | International Maritime Dangerous Goods |
| LC50 | Median lethal concentration |
| LD50 | Median lethal dose |
| LOAEL | Lowest Observed Adverse Effect Level |
| NOAEC | No-Observed Adverse Effect Concentration |
| NOAEL | No-Observed Adverse Effect Level |
| NOEC | No-Observed Effect Concentration |
| OECD | Organisation for Economic Co-operation and Development |
| OEL | Occupational Exposure Limit |
| PBT | Persistent Bioaccumulative Toxic |
| PNEC | Predicted No-Effect Concentration |
| RID | Regulations concerning the International Carriage of Dangerous Goods by Rail |
| SDS | Safety Data Sheet |
| STP | Sewage treatment plant |
| ThOD | Theoretical oxygen demand (ThOD) |
| TLM | Median Tolerance Limit |
| VOC | Volatile Organic Compounds |
| CAS-No. | Chemical Abstract Service number |
| N.O.S. | Not Otherwise Specified |
| vPvB | Very Persistent and Very Bioaccumulative |
| ED | Endocrine disrupting properties |

| Full text of H- and EUH-statements: |  |
| --- | --- |
| Acute Tox. 4 (Dermal) | Acute toxicity (dermal), Category 4 |
| Acute Tox. 4 (Oral) | Acute toxicity (oral), Category 4 |
| Aquatic Acute 1 | Hazardous to the aquatic environment – Acute Hazard, Category 1 |
| Aquatic Chronic 1 | Hazardous to the aquatic environment – Chronic Hazard, Category 1 |
| Eye Dam. 1 | Serious eye damage/eye irritation, Category 1 |
| H302 | Harmful if swallowed. |
| H312 | Harmful in contact with skin. |
| H314 | Causes severe skin burns and eye damage. |
| H315 | Causes skin irritation. |
| H318 | Causes serious eye damage. |
| H400 | Very toxic to aquatic life. |
| H410 | Very toxic to aquatic life with long lasting effects. |
| H412 | Harmful to aquatic life with long lasting effects. |
| Skin Corr. 1 | Skin corrosion/irritation, Category 1 |

Safety Data Sheet (SDS), EU