024362- Verto Concentrate Floor Cleaner 1L

Danger

| PROCESS STAGE | POTENTIAL RISK |  | CONTROL MEASURES |  | ACTION REQUIRED |
| --- | --- | --- | --- | --- | --- |
|  | DELIVERY |  |  |  |  |
| Packaged in 1 litre bottles in a cardboard carton. Delivered in single containers if required. | Risk of spillage if the container is dropped. Risk of injury to user if dropped. |  | Trained employees should handle only single containers at any given time. Take care not to drop the containers. Ensure that the cap is secure at all times. |  | Transfer containers to store immediately following delivery. Do not leave containers in areas where users of the building will come into contact with them. |
|  | STORAGE |  |  |  |  |
| Containers must be stored upright in a cool, well ventilated, dry storage area. Product must be kept stored in its original container. Store away from Oxidising materials. | Minimal risk of spillage if caps are firmly in place. Caps are not child proof. |  | Ensure that the product is stored away from any acid-based substances. Keep containers in secure storage areas. Store away from oxidising materials. Restrict access to storage areas to authorised personnel. |  | Use oldest stock first. Make regular checks to ensure that storage area is clean and tidy. Make regular checks to ensure that containers are not split or blown. |
|  | USAGE |  |  |  |  |
| Suitable for spray cleaning, damp mopping or scrubber dryer machine. Dilute as per label instructions. | Risk of eye contact. Risk of skin contact. Risk of skin irritation. Risk of ingestion. Risk of spillage onto floor and into drainage system. |  | Handle the containers with care. Spray product away from the body and face at all times. Ensure that the container is returned to store when not in use. Always read the manufacturer’s instructions prior to use. Never spray near foodstuffs. |  | Provide employees with PPE as follows: - Household rubber gloves - Eye protection - Protective clothing When mixing solutions, ensure that operatives add product to water Ensure that operatives are aware of location of First Aid facilities. |
|  | DISPOSAL |  |  |  |  |
| Empty containers must be rinsed thoroughly and disposed of as normal waste. Residual stocks may be disposed of into a foul sewer (sluice) using plenty of water. Recycle all containers wherever facilities are available. | Small risk of skin or eye contact. Risk of contamination with other substances. |  | Wash out the container using plenty of water. Do not store any other material in the empty container. Ensure that the product is exhausted before disposal. Ensure that the product is not disposed of via a rainwater sewer (ie drain outside the building.) |  | Dispose of contents/container in accordance with local regulations. |
| Name of Assessor: |  | Signed: |  | Print: |  |
| Position: |  | Date of Assessment: |  | Review Date: |  |

N.B. This risk assessment is offered as guidance only. It is your responsibility to ensure that it reflects the use of the product in your operation and amend accordingly.