Revision: 1

# SAFETY DATA SHEET APPLIANCE DESCALER

SECTION 1: Identification of the substance/mixture and of the company/undertaking

## 1.1. Product identifier

Product name 	APPLIANCE DESCALER

Product Code                                            024611

## 1.2. Relevant identified uses of the substance or mixture and uses advised against

Identified uses 	Cleaning agent. Acidic cleaner.

Uses advised against 	Do not use for personal cleansing.

## 1.3. Details of the supplier of the safety data sheet

| Supplier Contact person | Parc Global 2 Whitehall Point, Whitehall Road, Leeds, LS12 4RZ +44 (0)113 220 4080 info@parcglobal.com For content of safety data sheet: nfo@parcglobal.com |
| --- | --- |

## 1.4. Emergency telephone number

Emergency telephone 	+0113 220 4080

National emergency telephone For the emergency services - the ambulance, police and fire services - Tel: 999  / number When you need medical advice or treatment but it is not an emergency - Tel: 111

SECTION 2: Hazards identification

2.1. Classification of the substance or mixture

## Classification (EC 1272/2008)

| Physical hazards Health hazards Environmental hazards | Not Classified Skin Corr. 1B - H314 Eye Dam. 1 - H318 Not Classified |
| --- | --- |

### 2.2. Label elements

Hazard pictograms

| Signal word | Danger |
| --- | --- |
| Hazard statements | H314 Causes severe skin burns and eye damage. |
| Precautionary statements | P102 Keep out of reach of children. P264 Wash contaminated skin thoroughly after handling. P280 Wear protective gloves, eye and face protection. P301+P330+P331 IF SWALLOWED: Rinse mouth. Do NOT induce vomiting. P303+P361+P353 IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water or shower. P305+P351+P338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. P315 Get immediate medical advice/ attention. P405 Store locked up. P501 Dispose of contents/ container in accordance with local regulations. |
| Contains 2.3. Other hazards | phosphoric acid ... %, orthophosphoric acid ... % |
| Other hazards | None known |

SECTION 3: Composition/information on ingredients

#### 3.2. Mixtures

| phosphoric acid ... %, orthophosphoric acid ... % CAS number: 7664-38-2 EC number: 231-633-2 | 20-30% |
| --- | --- |
| Classification Skin Corr. 1B - H314 Eye Dam. 1 - H318 |  |

The full text for all hazard statements is displayed in Section 16.

SECTION 4: First aid measures

#### 4.1. Description of first aid measures

| Inhalation | Move affected person to fresh air at once. Rinse nose and mouth with water. Get medical attention if any discomfort continues. |
| --- | --- |
| Ingestion | Never give anything by mouth to an unconscious person. Do not induce vomiting. Rinse mouth thoroughly with water. Get medical attention if any discomfort continues. |
| Skin contact | Rinse immediately with plenty of water. Remove contaminated clothing. Get medical attention if irritation persists after washing. |
| Eye contact | Rinse immediately with plenty of water. Remove any contact lenses and open eyelids wide apart. Continue to rinse for at least 15 minutes. Get medical attention if irritation persists after washing. Show this Safety Data Sheet to the medical personnel. |

#### 4.2. Most important symptoms and effects, both acute and delayed

Skin contact	Causes burns.

Eye contact	Causes serious eye irritation.

4.3. Indication of any immediate medical attention and special treatment needed Notes for the doctor	No specific recommendations.

SECTION 5: Firefighting measures

#### 5.1. Extinguishing media

Suitable extinguishing media	The product is not flammable. Use fire-extinguishing media suitable for the surrounding fire. Foam, carbon dioxide or dry powder.

#### 5.2. Special hazards arising from the substance or mixture

Hazardous combustion	Thermal decomposition or combustion products may include the following substances: Oxides products	of carbon. Toxic gases or vapours.

#### 5.3. Advice for firefighters

Protective actions during	Control run-off water by containing and keeping it out of sewers and watercourses. firefighting

Special protective equipment	Wear positive-pressure self-contained breathing apparatus (SCBA) and appropriate protective for firefighters	clothing.

SECTION 6: Accidental release measures

6.1. Personal precautions, protective equipment and emergency procedures

Personal precautions	For personal protection, see Section 8. Contaminated surfaces will be extremely slippery.

#### 6.2. Environmental precautions

Environmental precautions	Collect and dispose of spillage as indicated in Section 13. Do not discharge into drains or watercourses or onto the ground.

#### 6.3. Methods and material for containment and cleaning up

Methods for cleaning up	Stop leak if safe to do so. Absorb in vermiculite, dry sand or earth and place into containers. Flush contaminated area with plenty of water. Avoid the spillage or runoff entering drains, sewers or watercourses.

#### 6.4. Reference to other sections

Reference to other sections	For personal protection, see Section 8. See Section 11 for additional information on health hazards. For waste disposal, see Section 13.

SECTION 7: Handling and storage

#### 7.1. Precautions for safe handling

| Usage precautions | Provide adequate ventilation. Avoid contact with skin and eyes. |
| --- | --- |
| Advice on general occupational hygiene | No specific hygiene procedures recommended but good personal hygiene practices should always be observed when working with chemical products. Wash at the end of each work shift and before eating, smoking and using the toilet. |

7.2. Conditions for safe storage, including any incompatibilities

Storage precautions	Store in tightly-closed, original container in a dry, cool and well-ventilated place.

#### 7.3. Specific end use(s)

Specific end use(s)	The identified uses for this product are detailed in Section 1.2.

SECTION 8: Exposure controls/Personal protection

8.1. Control parameters

### Occupational exposure limits

This product does not contain any hazardous materials with occupational exposure limits established by the region specific regulatory bodies.

phosphoric acid ... %, orthophosphoric acid ... %

Long-term exposure limit (8-hour TWA): WEL 1 mg/m³ Short-term exposure limit (15-minute): WEL 2 mg/m³ WEL = Workplace Exposure Limit.

#### 8.2. Exposure controls

Protective equipment

| Appropriate engineering controls | Provide adequate ventilation. |  |
| --- | --- | --- |
| Eye/face protection | Chemical splash goggles or face shield. |  |
| Hand protection | Chemical resistant protective gloves (EN374) |  |
| Other skin and body protection | Wear appropriate clothing to prevent repeated or prolonged skin contact. |  |
| Hygiene measures | When using do not eat, drink or smoke. Good personal hygiene procedures should be implemented. Wash hands and any other contaminated areas of the body with soap and water before leaving the work site. Use appropriate skin cream to prevent drying of skin. |  |
| Respiratory protection | Respiratory protection not required. |  |
| SECTION 9: Physical and chemical properties |  |  |  |
| 9.1. Information on basic physical and chemical properties Appearance Liquid. |  |  |  |
| Colour |  | Clear. Colourless. |  |
| Odour |  | Odourless. |  |
| pH |  | pH (concentrated solution): 1.9 |  |
| Relative density |  | 1.08 @ 20°C |  |
| Solubility(ies) |  | Soluble in water. |  |
| Explosive properties |  | Not applicable. |  |
| Comments |  | Information given is applicable to the product as supplied. |  |
| 9.2. Other information Other information |  | Not applicable. |  |
| SECTION 10: Stability and reactivity |  |  |  |

10.1. Reactivity

Reactivity	There are no known reactivity hazards associated with this product.

10.2. Chemical stability

Stability	Stable at normal ambient temperatures and when used as recommended.

#### 10.3. Possibility of hazardous reactions

Possibility of hazardous	Not applicable.

reactions

10.4. Conditions to avoid

Conditions to avoid	Avoid exposure to high temperatures or direct sunlight.

#### 10.5. Incompatible materials

Materials to avoid	No specific material or group of materials is likely to react with the product to produce a hazardous situation.

#### 10.6. Hazardous decomposition products

Hazardous decomposition	Does not decompose when used and stored as recommended. products

SECTION 11: Toxicological information

#### 11.1. Information on toxicological effects

| Toxicological effects Acute toxicity - oral | Information given is based on data of the components and of similar products. |
| --- | --- |
| Notes (oral LD₅₀) Acute toxicity - dermal | Based on available data the classification criteria are not met. |
| Notes (dermal LD₅₀) Acute toxicity - inhalation | Based on available data the classification criteria are not met. |
| Notes (inhalation LC₅₀) Skin corrosion/irritation | Based on available data the classification criteria are not met. |
| Skin corrosion/irritation Serious eye damage/irritation | Causes severe burns. |
| Serious eye damage/irritation Respiratory sensitisation | Causes serious eye damage. |
| Respiratory sensitisation Skin sensitisation | Not sensitising. Based on available data the classification criteria are not met. |
| Skin sensitisation Germ cell mutagenicity | Not classified. Based on available data the classification criteria are not met. |
| Genotoxicity - in vitro Carcinogenicity | Does not contain any substances known to be mutagenic. |
| Carcinogenicity Reproductive toxicity | Does not contain any substances known to be carcinogenic. |
| Reproductive toxicity - fertility | Does not contain any substances known to be toxic to reproduction. |

Specific target organ toxicity - single exposure

STOT - single exposure	Not classified as a specific target organ toxicant after a single exposure.

### Specific target organ toxicity - repeated exposure

STOT - repeated exposure	Not classified as a specific target organ toxicant after repeated exposure.

General information	This product has low toxicity.

Inhalation	No specific health hazards known.

Ingestion	May cause irritation. Symptoms following overexposure may include the following: Stomach pain. Nausea, vomiting. Diarrhoea.

| Skin contact | Skin irritation should not occur when used as recommended. Repeated exposure may cause skin dryness or cracking. |
| --- | --- |
| Eye contact | May cause temporary eye irritation. |

SECTION 12: Ecological information

12.1. Toxicity

Toxicity	Not considered toxic to fish

#### 12.2. Persistence and degradability

Persistence and degradability	There are no data on the degradability of this product. The surfactant(s) contained in this product complies(comply) with the biodegradability criteria as laid down in Regulation (EC) No. 648/2004 on detergents. Data to support this assertion are held at the disposal of the competent authorities of the Member States and will be made available to them at their direct request, or at the request of a detergent manufacturer.

12.3. Bioaccumulative potential

Bioaccumulative potential	No data available on bioaccumulation.

12.4. Mobility in soil

Mobility	The product is water-soluble and may spread in water systems.

#### 12.5. Results of PBT and vPvB assessment

Results of PBT and vPvB	This product does not contain any substances classified as PBT or vPvB.

assessment

#### 12.6. Other adverse effects

Other adverse effects	None known.

| SECTION 13: Disposal considerations |  |
| --- | --- |
| 13.1. Waste treatment methods General information | The generation of waste should be minimised or avoided wherever possible. |
| Disposal methods | The packaging must be empty (drop-free when inverted). Waste packaging should be collected for reuse or recycling. Dispose of waste to licensed waste disposal site in accordance with the requirements of the local Waste Disposal Authority. |
| SECTION 14: Transport information |  |

#### 14.1. UN number

UN No. (ADR/RID)	1805

UN No. (IMDG)	1805

UN No. (ICAO)	1805

UN No. (ADN)	1805

#### 14.2. UN proper shipping name

Proper shipping name	PHOSPHORIC ACID, SOLUTION (ADR/RID)

Proper shipping name (IMDG) PHOSPHORIC ACID, SOLUTION

Proper shipping name (ICAO)	PHOSPHORIC ACID, SOLUTION

Proper shipping name (ADN)	PHOSPHORIC ACID, SOLUTION

#### 14.3. Transport hazard class(es)

ADR/RID class	8

ADR/RID classification code	C1

ADR/RID label	8

IMDG class	8

ICAO class/division	8

ADN class	8

Transport labels

14.4. Packing group

ADR/RID packing group	III

IMDG packing group	III

ICAO packing group	III

ADN packing group	III

#### 14.5. Environmental hazards

Environmentally hazardous substance/marine pollutant No.

#### 14.6. Special precautions for user

IMDG Code segregation	1. Acids group

EmS	F-A, S-B

ADR transport category	3

Emergency Action Code	2R

Hazard Identification Number	80 (ADR/RID)

Tunnel restriction code	(E)

#### 14.7. Transport in bulk according to Annex II of MARPOL and the IBC Code

Transport in bulk according to	Not applicable. Annex II of MARPOL 73/78 and the IBC Code

SECTION 15: Regulatory information

#### 15.1. Safety, health and environmental regulations/legislation specific for the substance or mixture

National regulations	Control of Substances Hazardous to Health Regulations 2002 (as amended).

EH40/2005 Workplace exposure limits.

EU legislation	Commission Regulation (EU) No 2015/830 of 28 May 2015.

Regulation (EC) No 1272/2008 of the European Parliament and of the Council of 16 December 2008 on classification, labelling and packaging of substances and mixtures (as amended).

Regulation (EC) No 1907/2006 of the European Parliament and of the Council of 18 December 2006 concerning the Registration, Evaluation, Authorisation and Restriction of Chemicals (REACH) (as amended).

Commission Directive 2000/39/EC of 8 June 2000 establishing a first list of indicative occupational exposure limit values in implementation of Council Directive 98/24/EC on the protection of the health and safety of workers from the risks related to chemical agents at work (as amended).

Regulation (EC) No 648/2004 of the European Parliament and of the Council of 31 March 2004 on detergents (as amended).

#### 15.2. Chemical safety assessment

No chemical safety assessment has been carried out.

SECTION 16: Other information

Abbreviations and acronyms	PBT:  Persistent, Bioaccumulative and Toxic substance. used in the safety data sheet	vPvB:  Very Persistent and Very Bioaccumulative.

Revision	1

SDS number	24312

Hazard statements in full	H314 Causes severe skin burns and eye damage. H318 Causes serious eye damage. DIRECTIONS FOR USE

PRODUCT LOGO