SAFETY DATA SHEET

Verto Vytrex

SECTION 1: Identification of the substance/mixture and of the company/undertaking

1.1. Product identifier

Product name	Verto Vytrex

# 1.2. Relevant identified uses of the substance or mixture and uses advised against

Identified uses	Cleaning agent. Disinfectant.

Uses advised against	Do not use for personal cleansing.

# 1.3. Details of the supplier of the safety data sheet

| Supplier Contact person | Parc Global 2 Whitehall Point Whitehall Road Leeds LS12 4RZ +44 (0)113 220 4080 info@parcglobal.com For content of safety data sheet: info@parcglobal.com |
| --- | --- |

# 1.4. Emergency telephone number

Emergency telephone	+44 (0)113 220 4080

National emergency telephone For the emergency services - the ambulance, police and fire services - Tel: 999  / number	When you need medical advice or treatment but it is not an emergency - Tel: 111

SECTION 2: Hazards identification

# 2.1. Classification of the substance or mixture Classification (EC 1272/2008)

| Physical hazards | Not Classified |
| --- | --- |
| Health hazards | Skin Irrit. 2 - H315 Eye Dam. 1 - H318 |
| Environmental hazards | Aquatic Acute 1 - H400 Aquatic Chronic 2 - H411 |

# 2.2. Label elements

Hazard pictograms

| Signal word Hazard statements Precautionary statements Contains 2.3. Other hazards Other hazards | Danger H315 Causes skin irritation. H318 Causes serious eye damage. H400 Very toxic to aquatic life. H411 Toxic to aquatic life with long lasting effects. P102 Keep out of reach of children. P262 Do not get in eyes, on skin, or on clothing. P264 Wash contaminated skin thoroughly after handling. P305+P351+P338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. P337+P313 If eye irritation persists: Get medical advice/ attention. P501 Dispose of contents/ container in accordance with local regulations. N-(3-aminopropyl)-N-dodecylpropane-1,3-diamine None known |
| --- | --- |

SECTION 3: Composition/information on ingredients

# 3.2. Mixtures

| N-(3-aminopropyl)-N-dodecylpropane-1,3-diamine CAS number: 2372-82-9 EC number: 219-145-8 |  | 1-5% |
| --- | --- | --- |
| M factor (Acute) = 10 | M factor (Chronic) = 1 |  |
| Classification Acute Tox. 3 - H301 Skin Corr. 1B - H314 Eye Dam. 1 - H318 STOT RE 2 - H373 Aquatic Acute 1 - H400 Aquatic Chronic 1 - H410 |  |  |
| Alcohols, C9-11 ethoxylated, 6-8 EO |  | <1% |
| CAS number: 68439-46-3 | EC number: 614-482-0 |  |
| Classification Acute Tox. 4 - H302 Eye Dam. 1 - H318 |  |  |

The full text for all hazard statements is displayed in Section 16.

SECTION 4: First aid measures

# 4.1. Description of first aid measures

| Inhalation | Move affected person to fresh air at once. Rinse nose and mouth with water. Get medical attention if any discomfort continues. |
| --- | --- |
| Ingestion | Never give anything by mouth to an unconscious person. Do not induce vomiting. Rinse mouth thoroughly with water. Get medical attention if any discomfort continues. |
| Skin contact | Rinse immediately with plenty of water. Remove contaminated clothing. Get medical attention if irritation persists after washing. |

Eye contact	Rinse immediately with plenty of water. Remove any contact lenses and open eyelids wide

apart. Continue to rinse for at least 15 minutes. Get medical attention if irritation persists after washing. Show this Safety Data Sheet to the medical personnel.

# 4.2. Most important symptoms and effects, both acute and delayed

Skin contact	Prolonged contact may cause redness, irritation and dry skin.

Eye contact	May cause temporary eye irritation.

4.3. Indication of any immediate medical attention and special treatment needed Notes for the doctor	No specific recommendations.

SECTION 5: Firefighting measures

# 5.1. Extinguishing media

Suitable extinguishing media	The product is not flammable. Use fire-extinguishing media suitable for the surrounding fire. Foam, carbon dioxide or dry powder.

# 5.2. Special hazards arising from the substance or mixture

Hazardous combustion	Thermal decomposition or combustion products may include the following substances: Oxides products	of carbon. Toxic gases or vapours.

# 5.3. Advice for firefighters

Protective actions during	Control run-off water by containing and keeping it out of sewers and watercourses. firefighting

Special protective equipment	Wear positive-pressure self-contained breathing apparatus (SCBA) and appropriate protective for firefighters	clothing.

SECTION 6: Accidental release measures

6.1. Personal precautions, protective equipment and emergency procedures

Personal precautions	For personal protection, see Section 8. Contaminated surfaces will be extremely slippery.

# 6.2. Environmental precautions

Environmental precautions	Collect and dispose of spillage as indicated in Section 13. Do not discharge into drains or watercourses or onto the ground.

# 6.3. Methods and material for containment and cleaning up

Methods for cleaning up	Stop leak if safe to do so. Absorb in vermiculite, dry sand or earth and place into containers. Flush contaminated area with plenty of water. Avoid the spillage or runoff entering drains, sewers or watercourses.

# 6.4. Reference to other sections

Reference to other sections	For personal protection, see Section 8. See Section 11 for additional information on health hazards. For waste disposal, see Section 13.

SECTION 7: Handling and storage

# 7.1. Precautions for safe handling

| Usage precautions | Provide adequate ventilation. Avoid contact with skin and eyes. |
| --- | --- |
| Advice on general occupational hygiene | No specific hygiene procedures recommended but good personal hygiene practices should always be observed when working with chemical products. Wash at the end of each work shift and before eating, smoking and using the toilet. |

7.2. Conditions for safe storage, including any incompatibilities

Storage precautions	Store in tightly-closed, original container in a dry, cool and well-ventilated place.

# 7.3. Specific end use(s)

Specific end use(s)	The identified uses for this product are detailed in Section 1.2.

SECTION 8: Exposure controls/Personal protection

# 8.1. Control parameters Occupational exposure limits

This product does not contain any hazardous materials with occupational exposure limits established by the region specific regulatory bodies.

# 8.2. Exposure controls

Protective equipment

| Appropriate engineering controls | Provide adequate ventilation. |  |
| --- | --- | --- |
| Eye/face protection | No specific eye protection required during normal use. |  |
| Hand protection | For users with sensitive skin, it is recommended that suitable protective gloves are worn. |  |
| Other skin and body protection | Wear appropriate clothing to prevent repeated or prolonged skin contact. |  |
| Hygiene measures | When using do not eat, drink or smoke. Good personal hygiene procedures should be implemented. Wash hands and any other contaminated areas of the body with soap and water before leaving the work site. Use appropriate skin cream to prevent drying of skin. |  |
| Respiratory protection | Respiratory protection not required. |  |
| SECTION 9: Physical and chemical properties |  |  |  |
| 9.1. Information on basic physical and chemical properties Appearance Colour Odour pH Relative density Solubility(ies) Explosive properties Comments 9.2. Other information Other information |  | Liquid. Purple Characteristic. pH (concentrated solution): 11.0 ~ 1.003 @ 20°C Soluble in water. Not applicable. Information given is applicable to the product as supplied. Not applicable. |  |
| SECTION 10: Stability and reactivity |  |  |  |

10.1. Reactivity

Reactivity	There are no known reactivity hazards associated with this product.

10.2. Chemical stability

Stability	Stable at normal ambient temperatures and when used as recommended.

# 10.3. Possibility of hazardous reactions

Possibility of hazardous	Not applicable.

reactions

10.4. Conditions to avoid

Conditions to avoid	Avoid exposure to high temperatures or direct sunlight.

# 10.5. Incompatible materials

Materials to avoid	No specific material or group of materials is likely to react with the product to produce a hazardous situation.

# 10.6. Hazardous decomposition products

Hazardous decomposition	Does not decompose when used and stored as recommended. products

SECTION 11: Toxicological information

# 11.1. Information on toxicological effects

| Toxicological effects Acute toxicity - oral | Information given is based on data of the components and of similar products. |
| --- | --- |
| Notes (oral LD₅₀) | Based on available data the classification criteria are not met. |
| ATE oral (mg/kg) Acute toxicity - dermal | 3,333.33 |
| Notes (dermal LD₅₀) Acute toxicity - inhalation | Based on available data the classification criteria are not met. |
| Notes (inhalation LC₅₀) Skin corrosion/irritation | Based on available data the classification criteria are not met. |
| Skin corrosion/irritation Serious eye damage/irritation | Not classified. Based on available data the classification criteria are not met. |
| Serious eye damage/irritation Respiratory sensitisation | Based on available data the classification criteria are not met. |
| Respiratory sensitisation Skin sensitisation | Not sensitising. Based on available data the classification criteria are not met. |
| Skin sensitisation Germ cell mutagenicity | Not classified. Based on available data the classification criteria are not met. |
| Genotoxicity - in vitro Carcinogenicity | Does not contain any substances known to be mutagenic. |
| Carcinogenicity Reproductive toxicity | Does not contain any substances known to be carcinogenic. |
| Reproductive toxicity - fertility | Does not contain any substances known to be toxic to reproduction. |

Specific target organ toxicity - single exposure

STOT - single exposure	Not classified as a specific target organ toxicant after a single exposure.

# Specific target organ toxicity - repeated exposure

| STOT - repeated exposure | Not classified as a specific target organ toxicant after repeated exposure. |
| --- | --- |
| General information | This product has low toxicity. |
| Inhalation | No specific health hazards known. |
| Ingestion | May cause irritation. Symptoms following overexposure may include the following: Stomach pain. Nausea, vomiting. Diarrhoea. |
| Skin contact | Skin irritation should not occur when used as recommended. Repeated exposure may cause skin dryness or cracking. |
| Eye contact | May cause temporary eye irritation. |

Toxicological information on ingredients.

N-(3-aminopropyl)-N-dodecylpropane-1,3-diamine

| Acute toxicity - oral |  |
| --- | --- |
| ATE oral (mg/kg) Acute toxicity - oral | 100.0 Alcohols, C9-11 ethoxylated, 6-8 EO |
| ATE oral (mg/kg) Skin sensitisation | 500.0 |
| Skin sensitisation Reproductive toxicity | Guinea pig maximization test (GPMT) - Guinea pig: Not sensitising. |
| Reproductive toxicity - fertility | Two-generation study - NOAEL ≥ 250 mg/kg/day, Dermal, Rat P, F1 |
| Reproductive toxicity development | Developmental toxicity: - NOAEL: ≥ 250 mg/kg/day, Dermal, Rat |

SECTION 12: Ecological information

# 12.1. Toxicity

Toxicity	Not considered toxic to fish

Ecological information on ingredients.

N-(3-aminopropyl)-N-dodecylpropane-1,3-diamine

| Acute aquatic toxicity |  |
| --- | --- |
| LE(C)₅₀ | 0.01 < L(E)C50 ≤ 0.1 |
| M factor (Acute) Chronic aquatic toxicity | 10 |
| NOEC | 0.001 < NOEC ≤ 0.01 |
| Degradability | Rapidly degradable |
| M factor (Chronic) | 1 |

Alcohols, C9-11 ethoxylated, 6-8 EO

# Acute aquatic toxicity

Acute toxicity - fish	LC₅₀, 96 hours: 5 - 7 mg/l, Oncorhynchus mykiss (Rainbow trout)

Acute toxicity - aquatic	EC₅₀, 48 hours: 2.5 mg/l, Daphnia magna invertebrates

# 12.2. Persistence and degradability

Persistence and degradability	There are no data on the degradability of this product. The surfactant(s) contained in this product complies(comply) with the biodegradability criteria as laid down in Regulation (EC) No. 648/2004 on detergents. Data to support this assertion are held at the disposal of the competent authorities of the Member States and will be made available to them at their direct request, or at the request of a detergent manufacturer.

# 12.3. Bioaccumulative potential

Bioaccumulative potential	No data available on bioaccumulation.

Ecological information on ingredients.

Alcohols, C9-11 ethoxylated, 6-8 EO

Bioaccumulative potential	BCF: 237, Pimephales promelas (Fat-head Minnow)

# 12.4. Mobility in soil

Mobility	The product is water-soluble and may spread in water systems.

Ecological information on ingredients.

Alcohols, C9-11 ethoxylated, 6-8 EO

Surface tension	28.5 mN/m @ 20°C

# 12.5. Results of PBT and vPvB assessment

Results of PBT and vPvB	This product does not contain any substances classified as PBT or vPvB. assessment

Ecological information on ingredients.

Alcohols, C9-11 ethoxylated, 6-8 EO

Results of PBT and vPvB	This substance is not classified as PBT or vPvB according to current EU criteria.

assessment

# 12.6. Other adverse effects

Other adverse effects	None known.

| SECTION 13: Disposal considerations |  |
| --- | --- |
| 13.1. Waste treatment methods General information | The generation of waste should be minimised or avoided wherever possible. |
| Disposal methods | The packaging must be empty (drop-free when inverted). Waste packaging should be collected for reuse or recycling. Dispose of waste to licensed waste disposal site in accordance with the requirements of the local Waste Disposal Authority. |
| SECTION 14: Transport information |  |

# 14.1. UN number

UN No. (ADR/RID)	3082

UN No. (IMDG)	3082

UN No. (ICAO)	3082

UN No. (ADN)	3082

# 14.2. UN proper shipping name

Proper shipping name	ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S. (CONTAINS N-(3(ADR/RID)	aminopropyl)-N-dodecylpropane-1,3-diamine)

Proper shipping name (IMDG) ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S. (CONTAINS N-(3aminopropyl)-N-dodecylpropane-1,3-diamine)

Proper shipping name (ICAO)	ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S. (CONTAINS N-(3aminopropyl)-N-dodecylpropane-1,3-diamine)

Proper shipping name (ADN)	ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S. (CONTAINS N-(3aminopropyl)-N-dodecylpropane-1,3-diamine) 14.3. Transport hazard class(es)

ADR/RID class	9

ADR/RID classification code	M6

ADR/RID label	9

IMDG class	9

ICAO class/division	9

ADN class	9

Transport labels

14.4. Packing group

ADR/RID packing group	III

IMDG packing group	III

ICAO packing group	III

ADN packing group	III

# 14.5. Environmental hazards

Environmentally hazardous substance/marine pollutant

# 14.6. Special precautions for user

EmS	F-A, S-F

ADR transport category	3

Emergency Action Code	•3Z

Hazard Identification Number	90

(ADR/RID)

Tunnel restriction code	(-)

# 14.7. Transport in bulk according to Annex II of MARPOL and the IBC Code

Transport in bulk according to	Not applicable. Annex II of MARPOL 73/78 and the IBC Code

SECTION 15: Regulatory information

# 15.1. Safety, health and environmental regulations/legislation specific for the substance or mixture

| National regulations | Control of Substances Hazardous to Health Regulations 2002 (as amended). EH40/2005 Workplace exposure limits. |
| --- | --- |
| EU legislation | Commission Regulation (EU) No 2015/830 of 28 May 2015. Regulation (EC) No 1272/2008 of the European Parliament and of the Council of 16 December 2008 on classification, labelling and packaging of substances and mixtures (as amended). Regulation (EC) No 1907/2006 of the European Parliament and of the Council of 18 December 2006 concerning the Registration, Evaluation, Authorisation and Restriction of Chemicals (REACH) (as amended). Commission Directive 2000/39/EC of 8 June 2000 establishing a first list of indicative occupational exposure limit values in implementation of Council Directive 98/24/EC on the protection of the health and safety of workers from the risks related to chemical agents at work (as amended). Regulation (EC) No 648/2004 of the European Parliament and of the Council of 31 March 2004 on detergents (as amended). |

# 15.2. Chemical safety assessment

No chemical safety assessment has been carried out.

SECTION 16: Other information

| Abbreviations and acronyms used in the safety data sheet | PBT: Persistent, Bioaccumulative and Toxic substance. vPvB: Very Persistent and Very Bioaccumulative. |
| --- | --- |
| Revision | 1 |
| SDS number | 24283 |
| Hazard statements in full | H301 Toxic if swallowed. H302 Harmful if swallowed. H314 Causes severe skin burns and eye damage. H315 Causes skin irritation. H318 Causes serious eye damage. H373 May cause damage to organs through prolonged or repeated exposure. H400 Very toxic to aquatic life. H410 Very toxic to aquatic life with long lasting effects. H411 Toxic to aquatic life with long lasting effects. |

DIRECTIONS FOR USE

PRODUCT LOGO